Presenter

1. Operation

1.1 Before operation

Before using the Presenter, the user shall make sure the projector and the computer are connected well.

1) Equipment : PC, projector, LAN cable

2) Connection the PC and projector by LAN cable

Connect the projector to routers or switches of the LAN by direct or cross cable. If it fails to connect PC and projector by parallel cable, please switch to cross cables. The LAN cable quality shall be cat.5 or higher.

3) If the projector is connected well, one of the indicator LEDs of the projector will be light and ther other one shall be flash.

4) Make sure the PC and projector are in the same submask and the IP addresses are set well. For rhe reference, pleae see 3.LAN / WLAN connection. If the user have any problem, please contact your MIS.

5) The firewall and anti-virus application may block the execution of the Presenter, the user needs the right of the computer's administrator.

6) Please change the input source to ''LAN''.

1.2 Install Presenter

1) Get the application, Presenter, from your projector's agent.

2) Install the application in you PC. The application supports Window OS only, the user shall use WinXP, Win7 or higher.

3) Double click to execute the install application

4) Click the 'Next >' button to continue the installation and configure your PC's name when it is asked.

5) For the detail, please see 4.Install Presenter.

1.3 Operation

Bofore using the Presenter, please remember that the bandwith and the quality of network are the cirtical isuues for the Presenter, if the bandwith is not enough or the signal is not good enough, the user may not be able to project the image of the computer's desktop. If the user are using the wireless connection to project, please change to the wire connection, because of the environment may be not good enough in the moment, ex. Blue-tooth, the other wireless's interference.

1) Execute the Presenter by double click the shortcut.

2) The Presenter will display the main operation winodw.

3) Click the icon to start connection

4) If the projector IP address is known, please key in the IP address and click 'Connect' button. The user can get the projector IP from OSD.  Press Menu → Setup → Communications → LAN → IP address. In this example, the IP address is 10.2.24.253.

5) If the user doesn't know the projector IP address, please click 'Search' button directly. After waiting for a while, the avaiable projectors will be displayed. Click the projector you want to connect, and then click 'Connect directly'. In this example, the connected projector is ' Demo'.

Note, if only one projector is found, the Presenter will ask the user for connecting directly or not.

6) Select the login level. There is no password in defalut for the normal user. For the administrotor user, the password is 'admin'. In this example, the login level is normal user. Click the display region 'All' before pressing 'OK' button.

7) Some PC may popup a window to ask for make sure the change of the computer's setting by firewall or anti-virus application, it is normally because the resolution of the computer shall be change to 1280x768, otherwise it will fail to project.

8) When the user sees the following icon, please wait a while, the projector will project the desktop of the computer later.

9) For disconnection, please click 'stop all the connection' icon

2. Parameters

2.1) The main functions

The main window will appear similar to the following:

The functions with each icon are:

Search for network display

Stop/start displaying

Choose capture mode

Parameter settings

Network display management

Disconnect connection

2.2 Search for network display

In this function, the user can connect to a projector directly or find an avaiable projector to connection

2.2.1 Connect to the projector directly

If the user know the IP address of the target projector, type the IP address and press 'Connect' button directly. In this example, the IP address is '10.2.24.253'.

Directly : Project the computer's desktop.

Via webpage : Open the webpage if there is.

2.2.2 Search the projector

If the user doesn't know which projector can be used, press 'Search' button to find the projectors in the same section. After choosing one of the lists projector, click 'Connect directly' button to start connection. If only one projector is found, the Presenter will ask the user for connecting directly. If multiple projectors are found, the user shall choose one to connect.

In the following example, the Presenter finds 2 projector, and chooses the projector, Demo, to use.

Note : if the computer connects the projector with wire and wireless, only the wire connection will be found. It is not suggested that the user click 'Select all' to connects all of the projectors in the list, because the bandwith of the network may not enough to support, and the user will have problem to project.

2.3 Stop/start display

In this function, the user can decide to project the desktop on time or pause.

Presenter is not connected.

The screen projection is running, click this icon to pause.

The screen projection is paused, click this icon to run.

2.4 Choose capture mode

In this function, the user can choose the display area of the computer's desktop.

Full screen mode: The image of the whole screen is transferred

Fixed Size Mode: The part of the image selected by a frame is transferred. The user can drag the frame to the desired position.

Alterable Mode: The user can change the size and position of the frame by dragging any side or corner of the frame.

2.5 Parameter settings

In this function, the user can change the settings of the Presenter.

2.5.1 Basic setting

Language select : select the language

Region size of fixed size capture mode : the capture size in the fixed size mode.

Enable mirror driver when startup if there is : to enable the mirror dirver in the compuer to help the Presenter has better performce

Allow notification message popup : Enable or disable to pop up the notification message.

2.5.2 Advanced setting

Quality : the image quality to project. The higher quality choosed, the bigger bandwith it needs. If the image quality is too high and the network bandwith is not enough, the Presenter maybe not able to project, espectially for the wireless display.

YUV sample format : the format of the projection.

2.6 Network display management

In the function, the user changes the parameters of the connection.

2.6.1 Device management

The major items in the Device Management sheet are as following.

The display region of the projector ( see 2.8.2)

The adminstartor user

The normal user

The password setting

The password disable

disconnect this connection

connect to webpage if there is

For example,  if the user connection to a projector, the user may see the following figure.

2.6.2 Presenter management

Name : the name of the current Presenter, maximun15 capital letters.

2.7 Disconnect connection

In this function, the user can disconnect the connection directly.

No presenter connection

Presenter is connection, click this icon to quit the connection

2.8 Projection selection

2.8.1 The user can choose the display mode of the projection image by select display port.

All : Full screen of the projection image

L :  Left screen of the projection image

R : Right screen of the projection image

1 : Left top screen of the projection image

2 : Right top screen of the projection image

3 : Left bottom screen of the projection image

4 : Right bottom screen of the projection image

Background : hidden.

2.8.2 When the user connected already, the user change the project region too. Please go to 'Network display management' → 'Device management', and click the icon 'The display region of the projector'.

2.9 Login level

There are two levels for the users. The administrator user shall enter the password before login, the default password is 'admin'. The administrator user can change or assign the new passwords for the administrator user or the normal user. There is no default password for the normal users. The normal user can use the Presenter only, and shall enter the passsword the the administrator user assing it.

3. LAN / WLAN connection

3.1 LAN connnection

3.1.1 Directly connection by LAN

Please following the steps to setup the projector and PC.

1) Equipment

PC, projector, LAN cable

2) Connection the PC and projector by LAN cable directly.

Connect the projector to routers or switches of the LAN by direct or cross cable. If it fails to connect PC and projector by parallel cable, please switch to cross cables. The LAN cable quality shall be cat.5 or higher.

3) Power on the projector

If the projector is connected well, one of the indicator LEDs of the projector will be light and ther other one shall be flash.

4) Setup the IP address of the Projector

The user can use remote controller to press 'Menu' → 'Setup' → 'Communication' → LAN to set the IP address of the projector. Press 'Enter', 'Up', 'Down' buttons to modify the IP address. After that, the user shall press 'Apply' → 'Yes' to set it, and then while a while for the system works. In this example, the IP address is '192.168.0.100'. The subnet mask shall be '255.255.255.0'.

5) Setup the IP address of the computer

In the desktop of the computer, click 'Start' → Control Panel → Network and share center → Change adapter settings → Local Area connection → Internet Protocol Version 4 (TCP/IPv4) and click 'Properties' button.

Select the item, Use the fpllowing IP address. In this example, the IP address of the computer is set to '19.168.0.100' and the subnet mask is '255.255.255.0'. After that pressing 'OK' button and wait a while for the computer changes the IP.

6) Now, the directly connection is done. The user can access the projector by LAN network.

3.1.2 Add in an exist network environment

If there is an exist network environemnt, the user can use it if it is allowed. If the user doesn't know it, please contact your advisor before connecting, otherwise it may have problems of the network enviroment. The following case is just an example of the actions normally, and the computer operation system is Windows 7.

1) Equipment

Computer, Projector, 2 LAN cables

2) Connec the projector to the network with a LAN cable.

3) Set the projctor

Select 'DHCP' from 'Off' to 'On', and Select 'Apply' → 'Yes' to change the setting. It may take a while because the DHCP server need to assign a new IP for the projector. In this example, it is assigned to '10.2.24.253'.

4) Connect the computer to network

5) Change the computer setting

In the desktop of the computer, click 'Start' → Control Panel → Network and share center → Change adapter settings → Local Area connection → Internet Protocol Version 4 (TCP/IPv4) and click 'Properties' button. Select the item 'obtain an 'IP address automatically' and click 'OK' button.

6) Normally the settings are finished. If the user can not access the projector, please contact your MIS.

3.2 Connect with wieless

The steps to connect the projector by wireless is as following

- Insert the supplied USB dongle into USB port of the projector

- Waiting for two minutes before opening the OSD to the page 'WLAN'

The WLAN shall be enabled before using, and the user can find the SSID in this Page, too. If it is not enabled, please change the item, Enable, form 'Off' to 'On' and then then waiting 2 minutes for system works. In this example, the SSID is 'Demo'.

3) Opem the computer wilress connection list

The computer shall find the connection, Demo, as below figure, click 'Connect' button to start connecting. If the user can not find it, please  refresh the list or re-open the WLAN again.

Note: If the signal is less than 4 digits, the signal strength or the single quality may be not good enough. The user may have problem to use wireless to access the projector.

4) The user can use wireless connection now.

If the user can not open the webpage, there are two things to check.

4.1) Signal issue

Try to disconnect the connection, changing the position of the computer or the projector and then connect it again. If it is still not workable, maybe the wireless environment is too dirty currently.

4.2) Firewall or anti-virus application

Disconnect it and change the settings of the computer. In following figure, the firewall is opened for safety reason, but the wireless connection may have problem.

4. Install Presenter

1) Get the Presenter setup application from your manufacturer or agent.

2) Execute Presenter setup application by double click icon

3) Click 'Next >' button to the next step

4) Click 'I Agree' to the next step

5) Click 'Next >' to next step

6) Click 'Next >' to next step

7) Key in the Name and Click 'Next >' to the next step

8) Click 'Install' to install

9) Click 'Finish' to reboot the computer

10) After rebooting, the user will find a shortcut on your desktop.